| Albany County Office of Code Enforcement |  |  |
| --- | --- | --- |
| 112 State Street Rm. 830 Albany, New York 12207 |  | Phone (518) 447-5638 Fax (518) 447-5587 |

- Operating Permit

- General Requirements for the use of Mobile Lasers

Application for permit to operate a display Mobile Lasers in accordance with the terms of the New York State Uniform Fire Prevention and Building Codes, New York State Department of labor, and the Ordinances of the County of Albany and the requirements of the County of Albany Office of Code Enforcement. Certified mobile laser operators shall operate mobile lasers in compliance with the requirements of 12 NYCRR Part 50.

The following must be provided in order to constitute a completed application:

- Complete and return the application for  Operating permit for the use of Mobile Lasers, CEO90

- The Name of the Organization sponsoring the display together with the names

of the persons actually in charge of firing the display.

- Evidence of financial responsibility, including Insurance Carrier, amount of and

policy number of Insurance Coverage and Policy Holder the minimum amount

to be one million dollars, Proof of NYS Disability and NYS Workers Compensation insurance See attached

“Insurance Requirements” CEO73

- Date, time and duration of the displays.

- Exact location of the display.

- Provide qualifications of the Laser Safety Officer.

- Provide a description setting forth the, experience, and physical characteristics of the

Technician and their Assistants and a definition of the general responsibilities of assistants.

- The Laser Safety officer.

- The name of the designated individual to supervise the operation of the laser.

- The number, names, and ages of all assistants who are to be present.

- Provide a copy applicable NYS Department of Labor Certificate of Competence held by the

operator and information regarding the rejection, suspension or revocation of any previous certificate.

- Provide a copy of the New York State Department of Labor Registration of laser Installations and Mobile lasers.

- Provide a copy of the Laser Safety Officers certificate of competence issued by the New York State Department of labor.

- Provide copy of New York State Department of labor- Notice of Intent to Use Mobile Lasers at Temporary Job Site.

Laser Safety Officer.

The owner of any laser installation or mobile laser shall designate a laser safety officer who shall establish

and administer a laser radiation safety program which is in compliance with New York State Department of Labor Part 50 lasers  (rule). The laser safety officer for a high-intensity mobile laser may be the certified operator of such laser. The laser safety officer for one or more low-intensity lasers at a given site or location may be a certified mobile laser operator (Class A). When a laser safety

officer does not personally supervise the safety aspects of laser operation, he may designate another individual, who

has been sufficiently instructed and trained by the laser safety officer in appropriate safety techniques, to personally

supervise the operation of a laser. The laser safety officer or any individual he designates to supervise the safety

aspects of laser operation may also operate laser equipment subject to the other applicable provisions of  New York State Department of Labor Part 50 Lasers.

(b) Categories of certificates of competence. There shall be two categories of certificates of competence

issued to operators of mobile lasers, as follows:

(1) Class A certificates. The holder of a Class A certificate of competence may operate any low intensity

mobile laser.

(2) Class B certificates. The holder of a Class B certificate of competence may operate any high intensity

or low-intensity mobile laser.

Carrying certificate. Each certified mobile laser operator shall carry his certificate of competence on his

person whenever he is working with or operating any mobile laser. Failure to produce such certificate upon request

by the Code Enforcement Officer shall be considered presumptive evidence that the operator is not certified.

- Laser Radiation Protection Program.

Before the performance of any production, the permit applicant shall submit a plan for the use of lasers to the County of Albany Office of Code Enforcement.

After a permit has been granted, the permittee shall keep the plan available at the site for Code Enforcement officers or other designated agents of the Office of Code Enforcement.

Any addition of pyrotechnics to a performance or any significant change in the presentation of pyrotechnics shall require approval by the Office of Code Enforcement.

A diagram of the grounds or facilities where the production is to be held. This diagram shall show the point at which the mobile lasers are to be used.

Designation of laser radiation area.

(a) Laser radiation symbol. The laser radiation symbol, as shown below, shall be used to designate lasers

and laser radiation areas. The symbol shall be in red and the background area shall be yellow or white.

LASER RADIATION SYMBOL

(b) Laser radiation area. Except as below provided, each person who possesses a laser shall post

conspicuously at the entrance to and inside of each laser radiation area a sign constructed of durable material bearing

the laser radiation symbol and the words "CAUTION" or "DANGER" and "LASER RADIATION AREA". Such

words shall be in black letters at least one inch in height.

Exception: No such area need be posted with any sign or equipped with any required control device solely because of

the presence therein of any laser for any period of not more than eight hours, provided, however, that

during such period the laser safety officer or a designated individual is constantly in attendance and such

safety officer or designated individual shall take all precautions necessary to prevent any individual from

receiving any exposure to laser radiation that exceeds the applicable limits permitted by this Part (rule)

and from receiving any other injury from any other hazard associated with the operation of any laser.

Such area shall be subject to the control of the owner of such laser.

Safeguarding and disposing of lasers.

(a) Safeguarding. Each laser shall be safeguarded against unauthorized use.

Designation of lasers.

(a) Every laser not otherwise exempt by this Part (rule) shall have a label so attached as to be clearly visible

and such label shall contain the following information:

(1) The laser radiation symbol in red.

(2) The words "CAUTION" or "DANGER" and "LASER" in black letters.

(3) The wave length or lengths of the emitted laser radiation.

(4) The maximum output power or energy density or densities of the emitted laser radiation.

(5) The divergence of the emitted laser radiation in its lowest order transverse mode.

(b) Such required information shall be legibly stamped, etched or otherwise permanently marked on a yellow

or white label with letters and numbers not less than three millimeters (one- eighth inch) in height where practicable.

Associated hazards.

In addition to the protection against laser radiation, protection shall also be provided against any associated

hazards which may be present during the operation of any laser. Such associated hazards include but are not limited

to the following:

- Electrical hazards. Any laser and its associated electrical system may present electrical hazards. Every

laser, including all associated electrical equipment, shall be so designed, constructed, installed and maintained as to

minimize the possibility of electrical hazards.

- Fire hazards. Every laser and every laser installation shall be designed, constructed, installed, operated

and maintained so as to eliminate or reduce any fire hazard. In addition, every laser radiation area shall be

maintained clear of any unnecessary materials in order to minimize any fire hazard.

- Explosion hazards. Lasers and associated equipment may present explosion hazards. Such equipment

shall be protected with explosion shields to prevent injury to individuals from such hazards.

Application must be made at least fifteen days in advance of the date set for the display. Upon receipt of a completed application, the Office of Code Enforcement shall make, or cause to be made, and inspection of the site of the proposed display for the purpose of determining whether of a permit would be consistent with public safety.

- The Office of Code Enforcement shall, within fifteen days of the date of the receipt of a completed application, either grant or deny such application.

- 5/13       									 CEO89

- Pg. 2